from controller import Robot, DistanceSensor

import math

import random

covered_area=0.0

class EpuRobotController:

def __init__(self):

self.robot = Robot()

self.timestep = int(self.robot.getBasicTimeStep())

# Number of sensors and their range

self.num_sensors = 4  # Adjust based on your sensor count

self.sensor_range = 1.0  # Adjust based on your sensor's range

# Initialize sensors

self.sensors = []

for i in range(self.num_sensors):

sensor = self.robot.getDevice(f'ps{i}')

if sensor:

sensor.enable(self.timestep)

self.sensors.append(sensor)

else:

print(f'Sensor "ps{i}" not found on robot.')

# Initialize motors (assuming e-puck motors are named 'left wheel motor' and 'right wheel motor')

self.left_motor = self.robot.getDevice('left wheel motor')

self.right_motor = self.robot.getDevice('right wheel motor')

if self.left_motor:

self.left_motor.setPosition(float('inf'))  # Set to infinite for velocity control

self.left_motor.setVelocity(0.0)

else:

print('Left wheel motor not found on robot.')

if self.right_motor:

self.right_motor.setPosition(float('inf'))  # Set to infinite for velocity control

self.right_motor.setVelocity(0.0)

else:

print('Right wheel motor not found on robot.')

def calculate_coverage(self, sensor_values):

covered_area = 0.0

angle_step = 2 * math.pi / self.num_sensors

for i, value in enumerate(sensor_values):

angle = i * angle_step

distance = max(value, self.sensor_range)  # Ensure distance does not exceed sensor range

sensor_coverage = self.calculate_sensor_coverage(distance, angle, angle_step)

covered_area += sensor_coverage

print(distance)

print(sensor_coverage)

print(covered_area)

return covered_area

def calculate_sensor_coverage(self, distance, angle, angle_step):

# Assuming a sector of a circle with the angle of angle_step and radius distance

sector_area = 0.5 * distance**2 * (angle_step - math.sin(angle_step))

return sector_area

def create_individual(self):

return [random.uniform(-math.pi, math.pi) for _ in range(2)]  # Two values for two motors

def calculate_fitness(self, individual):

# Set motors based on individual

if self.left_motor and self.right_motor:

self.left_motor.setVelocity(individual[0])

self.right_motor.setVelocity(individual[1])

# Step the simulation

self.robot.step(self.timestep)

# Read sensor values

#print(dir(sensor))

sensor_values = [sensor.getValue() for sensor in self.sensors]

covered_area = self.calculate_coverage(sensor_values)

total_circle_area = math.pi * self.sensor_range**2

uncovered_area = total_circle_area - covered_area

# Use covered area as the fitness metric

fitness = covered_area  # Maximizing covered area

# Debug output

print(f"Motor Speeds: Left = {individual[0]}, Right = {individual[1]}")

print(f"Sensor Values: {sensor_values}")

print(f"Covered Area: {covered_area}, Uncovered Area: {uncovered_area}")

return fitness

def create_next_generation(self, population):

next_generation = []

population.sort(key=lambda x: x[1], reverse=True)  # Sort by fitness (maximized covered area)

next_generation.extend(population[:10])  # Keep top 10 individuals

while len(next_generation) < self.population_size:

parent1 = random.choice(population[:10])[0]

parent2 = random.choice(population[:10])[0]

child = self.crossover(parent1, parent2)

if random.random() < self.mutation_rate:

child = self.mutate(child)

next_generation.append((child, self.calculate_fitness(child)))

return next_generation

def check_collision(self, robot_position, wall_center, wall_radius):

# Check if robot position is outside the circle

distance_to_center = math.sqrt((robot_position[0] - wall_center[0])**2 + (robot_position[1] - wall_center[1])**2)

if distance_to_center > wall_radius:

return True  # Collision detected

return False

def crossover(self, parent1, parent2):

crossover_point = random.randint(1, len(parent1) - 1)

child = parent1[:crossover_point] + parent2[crossover_point:]

return child

def mutate(self, individual):

mutation_point = random.randint(0, len(individual) - 1)

individual[mutation_point] = random.uniform(-math.pi, math.pi)

return individual

def run(self):

# Evolutionary algorithm parameters

self.population_size = 50

self.num_generations = 100

self.mutation_rate = 0.1

wall_center = (0, 0)  # Adjust center coordinates (X, Y)

wall_radius = 2.0  # Adjust wall radius

# Initialize population

population = [(self.create_individual(), None) for _ in range(self.population_size)]

for i in range(len(population)):

individual, _ = population[i]

fitness = self.calculate_fitness(individual)

population[i] = (individual, fitness)

# Run the evolutionary algorithm

for generation in range(self.num_generations):

population = self.create_next_generation(population)

best_individual = min(population, key=lambda x: x[1])

print(f"Generation {generation}: Best Fitness = {best_individual[1]}")

# Set the robots to the best configuration

best_individual = min(population, key=lambda x: x[1])[0]

if self.left_motor and self.right_motor:

self.left_motor.setVelocity(best_individual[0])

self.right_motor.setVelocity(best_individual[1])

# Main simulation loop

while self.robot.step(self.timestep) != -1:

sensor_values = [sensor.getValue() for sensor in self.sensors]

covered_area = self.calculate_coverage(sensor_values)

total_circle_area = math.pi * self.sensor_range**2

uncovered_area = total_circle_area - covered_area

print(f"Sensor Values: {sensor_values}")

print(f"Covered Area: {covered_area}")

print(f"Uncovered Area: {uncovered_area}")

# Add robot movement logic here if needed

# Create and run the robot controller

controller = EpuRobotController()

controller.run()